Title:  Isolation of Serum, Plasma, & Peripheral Blood                     Mononuclear Cells (PBMCs) from Human Blood

Solid Tumor Tissue Bank and Repository

Standard Operating Procedure

Page:  1 of 5

SOP #:  M01                                   Revision Level:  2.0		         Effective Date:  09/01/2017

Author: Jacqueline Mudd

# 1.  PURPOSE

This protocol describes the materials and methods for the isolation of serum, plasma, and peripheral      blood mononuclear cells [PBMCs] from human blood.

# 2.  SCOPE

This procedural format is used for all blood specimens procured for processing and deposition into the HPB Tissue Bank and Registry within the Section of HPB Surgery.

# 3.  MATERIALS

3.1      Ficoll-Paque Plus [GE Healthcare catalog #17-1440-02]  Store at room temperature and protect from light

3.2      1X Dulbecco’s Phosphate Buffered Saline – D-PBS, Sterile, [-] Calcium &  [-] Magnesium

[Gibco Cat #14190-136]  Store at room temperature.

3.3       Fetal Bovine Serum – FBS [Gibco Cat #26140-079] Store at -20C until use.

3.4      Freezing Media [Fetal Bovine Serum + 10% Dimethyl Sulfoxide] Store at -20C until use; do not  refreeze.

3.5      Dimethyl Sulfoxide – DMSO [Sigma Aldrich Cat #D8418] Store at room temperature.

3.6      Trypan Blue Solution [Fisher Scientific Cat #SV3008401] Store at room temperature

3.7      Hemocytometer [Fisher Scientific Cat #0267110]

3.8      Nalgene Cryogenic Vials [Fisher Scientific Cat #03-337-7D]

Title:  Isolation of Serum, Plasma, & Peripheral Blood                     Mononuclear Cells (PBMCs) from Human Blood

Solid Tumor Tissue Bank and Repository

Standard Operating Procedure

Page:  2 of 5

SOP #:  M01                                 Revision Level:  2.0		         Effective Date:  09/01/2017

Author: Jacqueline Mudd

3.9      Microcentrifuge Tubes, 2 ml, with o-ring cap [Fisher Scientific Cat #02-682-558]

3.10     Conical Tube, Sterile, 50 ml [Fisher Scientific Cat #14-432-22]

3.11     Conical Tube, Sterile, 15 ml [Fisher Scientific Cat #14-959-49B]

3.12     Sorvall ST-16 Benchtop Centrifuge with swinging bucket rotor

3.13     Nalgene ‘Mr. Frosty’ Freezing Container [Fisher Scientific Cat #15-350-50]

3.14    LabXpert Perm Nylon Cloth Label (X-78-499), box label, 150 each

[Fisher Scientific Cat #11-78-499]

3.15    LabXpert Hi-Perf Lab Polyester Label (XSL-86-461), cryogenic tube label, 150 each

[Fisher Scientific Cat #11-878-134]

3.16    LabXpert Perm Nylon Cloth Label (X-120-499), 2 ml tube label, 225 each

[Fisher Scientific Cat #11-878-136]

3.17    Serological Pipettes, 2 ml, sterile, case of 500 [Fisher Cat #13-678-11C]

3.18    Serological Pipettes, 5ml, sterile, case of 200 [Fisher Cat #13-678-11D]

3.19    Serological Pipettes, 10 ml, sterile, case of 200 [Fisher Cat #13-678-11E]

3.20    Serological Pipettes, 25ml, sterile, case of 200 [Fisher Cat #13-678-11]

3.21    Vacutainer Tubes (BD Cat#367880), 10 ml, with Lithium Heparin [Fisher Scientific Cat #02-689-7]

3.22    Vacutainer Tubes (BD Cat#367815), 6 ml, serum, 100 each [Fisher Scientific Cat #02-683-94]

Title:  Isolation of Serum, Plasma, & Peripheral Blood                     Mononuclear Cells (PBMCs) from Human Blood

Solid Tumor Tissue Bank and Repository

Standard Operating Procedure

Page:  3 of 5

SOP #:  M01                                 Revision Level:  2.0		         Effective Date:  09/01/2017

Author: Jacqueline Mudd

# 4.  PROCEDURE

4.1     Please use precautions for processing blood samples as follows:

- All processing done in a biosafety cabinet

- Equipment used approved for processing human specimens

- Appropriate PPE worn by staff [lab coat, gloves, etc.]

4.2      Mix all blood tubes and process within [1] hour after receipt from the OR

4.3      In a 50 ml sterile conical tube, gently layer the heparinized blood on top of 25 ml of Ficoll-Paque   Plus at room temperature

4.4      Centrifuge both the 6 ml serum tube and the Ficoll-Paque Plus gradient tube at 400 x g for 30 minutes at room temperature.  Acceleration and brake both slowed to setting “2”

4.5      Remove serum and generate 6 x 0.4 ml aliquots in 2 ml microcentrifuge tubes with o-ring cap

- Place tubes on ice.  Record number of aliquots and respective volume on specimen log form

4.6      Remove upper plasma layer from Ficoll-Paque Plus gradient tube and generate 6 x 1.5 ml aliquots in 2 ml microcentrifuge tubes with o-ring cap

- Place tubes on ice.  Record number of aliquots and respective volume on specimen log form

4.7      Discard most of remaining plasma from Ficoll-Paque Plus gradient tube.  Leave ~ 0.5 cm above the buffy coat interphase

4.8       Remove the buffy coat interphase between the Ficoll-Paque Plus and plasma layers with a 5 ml serological pipette and transfer to a new sterile 50 ml conical tube.  Minimize acquisition of Ficoll-Paque Plus gradient layer.

4.9       Discard remaining fluids into appropriate biohazard container

Title:  Isolation of Serum, Plasma, & Peripheral Blood                     Mononuclear Cells (PBMCs) from Human Blood

Solid Tumor Tissue Bank and Repository

Standard Operating Procedure

Page:  4 of 5

SOP #:  M01                                 Revision Level:  2.0		         Effective Date:  09/01/2017

Author: Jacqueline Mudd

4.10      Add room temperature 1X D-PBS (Ca & Mg free) to the PBMC containing buffy coat, fill to 45 ml and invert to mix

4.11      Pellet PBMCs at 300 x g for 10 minutes at room temperature.  Acceleration and brake both increased to setting “9”

4.12      Decant and discard the 1X D-PBS (Ca & Mg free) supernatant

4.13      Add 10 ml room temperature 1X D-PBS (Ca & Mg free) and resuspend pellet to single-cell suspension with a 2 ml serological pipette

4.14      Count live cells using Trypan blue exclusion procedure with hemocytometer and determine total number of cell isolates [ # cells / ml x 10 ml total volume]

4.15       Re-pellet PBMCs at 300 x g for 10 minutes at room temperature.  Acceleration and brake both increased to setting “9”

4.16      Decant and discard the 1X D-PBS (Ca & Mg free) supernatant

4.17     Resuspend PBMCs in sterile freezing media [FBS + 10% DMSO] to a 5 x 106 cells / ml concentration and generate 9 x 1 ml aliquots in Nalgene Cryogenic Vials

- Place tubes on ice.  Record number of aliquots and respective volume on specimen log form

4.18      Transfer PBMC aliquots to a Mr. Frosty freezing container and place in a -80C freezer for 24 hours.

- Transfer from -80C to a  liquid nitrogen tank after 24 hour period

Title:  Isolation of Serum, Plasma, & Peripheral Blood                     Mononuclear Cells (PBMCs) from Human Blood

Solid Tumor Tissue Bank and Repository

Standard Operating Procedure

Page:  5 of 5

SOP #:  M01                                 Revision Level:  2.0		         Effective Date:  09/01/2017

Author: Jacqueline Mudd

4.19     Transfer serum and plasma vials from ice to -80C freezer

4.20      All cryogenic and microcentrifuge tubes will be labelled with the following information:

- HPB TB&R

- Date

- ID#

- Sample Type [Serum, Plasma, PBMCs]

- IRB ID#:  201108117